# TCSS371 – Machine Organization
Assignment 2 –Digital Logic Structures

### 20 Points

Group Members’ Names:  _______Jonathan Sung____________________________

Purpose: This homework will test your understanding of the digital logic structures concepts that we covered from Chapter 3 of the textbook.

Student Learning Outcomes: The following student-learning outcomes are addressed in this assignment:

- explain the function of basic digital logic circuits

- design simple logic circuits after specifying their behavior using a truth table

IMPORTANT NOTE: Circuit files created in older versions of Logisim usually do NOT work correctly when opened in Logisim Evolution. It is important that you user the current version of Logisim Evolution. When I grade the assignment I will use the current version of Logisim Evolution.

- (Circuits) Create the following circuits in Logisim Evolution as you have seen the instructor do in the lectures. DO NOT use the built-in MUX, ADDER or D-Latch for any part of question 1.  You must build the circuits using AND, OR and NOT logic gates. Each circuit should be submitted in a different circuit of the same Logisim file (NOT the main circuit).

The main circuit should not have any circuit drawings. It should just have your name(s) displayed using the text tool.

  - A 10-to-1 mux using basic gates (AND, OR, NOT). Label inputs and outputs and provide a truth table for this circuit on the circuit diagram.

  - An 8-bit full adder. Create a 1-bit adder first using basic gates (AND, OR, NOT) and then use it as a block to create an 8-bit adder. Label the inputs and outputs. A truth table is not required.

NOTE: You can use a constant set to zero (from the ‘wiring’ section in Logisim) for the carry in to the least significant bit.

  - An 8-bit D-Latch. Create a 1-bit gated D-Latch using basic gates (AND, OR, NOT) and then reuse it as a block to create an 8-bit D-Latch.

Label the inputs and outputs. A truth table is not required.

- (Truth table to circuit) Create a truth table for a 4-bit input in which the output will be a 1 if the decimal number represented by the input bit pattern is divisible by 3 and/or 5. Assume that ABCD represent the most significant to least significant bits of a binary input pattern.

Also generate the gate-level logic circuit, using the implementation algorithm we discussed in lecture. Use Logisim to create the circuit.

This truth table is the only deliverable that will be in the written submission.

Make sure that you name(s) are on your submission.

| A | B | C | D | Z (Output) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

- (ALU) Design a 4-bit ALU. Below is a block diagram of a simplified ALU. The three select lines (S0, S1, and S2) should determine which operation the ALU performs from the set {NOTA, NOTB, AND, NAND, OR, NOR, XOR, ADD}

ALU

A

BA

OUT

COUT

CIN

S2

S0

S1

All the components that you need to use are already in Logisim. You will need the basic gates, inputs, outputs, adder, multiplexer, etc. to complete the circuit. You don’t need to use transistors or build the adder, multiplexer, etc. instead raise your abstraction level to gates and built-in components. Provide a table on your circuit diagram explaining which pattern of select bits performs each operation.

Failure to provide a table of inputs to outputs on your circuit diagram will reduce the score on this question by half.

Here’s how each operation works:

NOT A and NOT B operations - Only a single input is used in these operations (NOT A just uses the A input for example). If the A input is 0011 and NOT A is the selected operation, then OUT will be 1100.

ADD operation – When the ADD operation is chosen, the A, B, and Cin inputs are used to determine OUT and Cout.

Other operations – The A and B inputs are used to perform bitwise AND, NAND, OR, NOR, and XOR operations. The output of the selected operation is sent to OUT. For example: If A is 1010, B is 1111, and the XOR operation is chosen using the select bits, then the output will be 0101.

Make sure that Cout is 0 unless the ADD operation is chosen and the ADD operation results in a carry out. For example, when A is 0100, B is 1111, and the ADD operation is chosen, then Cout should be 1; however, if any other function is chosen then Cout should be 0. How do you make sure that Cout is 0 when we are not performing an ADD operation?

NOTE: Cout should never be an ERROR signal.

Build the Logisim circuit in steps. In addition to demonstrations in class, the built-in tutorial in Logisim can help you to understand how to build circuits using Logisim.

Group Submission guidelines: Submit your homework on Canvas as assignment2.docx (or pdf or doc) for the truth table and assignment2.circ for all the circuits. All circuits must be in a single .circ file. Your name or names must be in both submissions.

Grading Criteria: This assignment is worth 20 points and will be graded using the rubric below.

|  | Points |
| --- | --- |
| Circuits for Mux, Adder, Latch | 5 Points total All the sub problems are solved correctly using basic logic gates (AND, OR, NOT) Mux – 2.0 Adder – 1.5 Latch – 1.5 |
| Truth table and Logisim circuit for the truth table | 5 Points total (2.5 Points) Truth table has the correct values for all the inputs and output (2.5 Points) Circuit is implemented correctly for the truth table |
| 4-bit ALU | 10 Points total 0.5 Points each for !A and !B 5 Points - 1 Point for each (other than the adder) 2 Points for the adder 1 Point for making sure that Cout is 1 only when add is used 1 Point for labeling of inputs, select lines, and outputs (-5 Points if no legend is provided on the circuit diagram) |